# Part 3 – Technical Requirements

## Supply, Delivery and Installation, of a Point-to-Point Blood Transportation System for Indira Gandhi Memorial Hospital (IGMH) and Dharumavantha Hospital (DH)

## Section 1 – Introduction

To establish an automated specimen transportation network connecting designated clinical locations within IGMH to the Main Laboratory at Dharumavantha Hospital to improve laboratory turnaround time, reduce manual handling, improve specimen traceability and enhance patient care. The need for improved specimen logistics and laboratory turnaround is consistent with the hospital justification and technical evaluation documents.

Bidders shall carefully review this bidding document and submit bids in strict accordance with the instructions below. Failure to submit the required documents, forms, technical evidence, securities, or declarations may result in the bid being considered non-responsive.

- Bidders shall be legally registered entities eligible to supply, install, commission and maintain the proposed system.

- A site visit may be arranged by IGMH. Bidders remain responsible for verifying site conditions, routing constraints, installation requirements and integration needs.

- All documents shall be signed by an authorized representative of the bidder. Any correction, alteration or overwrite shall be initialed. Bidders shall ensure that the submitted technical evidence clearly demonstrates compliance with every requirement in this document.

# Section 2– Scope of Work

The Contractor shall:

### Supply

- Eight (8) Sending Stations

- Four (4) Receiving Stations

- Central Monitoring System

- Transport Network Infrastructure

- Controllers and Communication Equipment

- Software Licenses

- All cabling, tubing, brackets, supports and accessories

### Services

- Site survey

- Engineering design

- Installation

- Testing and commissioning

- Training

- Warranty support

- Preventive maintenance

## Proposed Areas for Installation

Sending Stations

- Main Laboratory (Central Hub/Phlebotomy/OPD Nurses Station) - IGMH

- Emergency Department (Main ER and Pediatric ER)

- Inpatient Ward Area – 1st Floor (Main Theatre Complex, ICU, NIVU, Medical Ward, PVT 2, NCC, Pvt 1, Isolation Ward)

- Inpatient Ward Area – 2nd Floor (Paediatric Ward, Medical Ward Annex, ENT, Ophathalmology, PVR 3, PVT 4)

- Multi-Specialty Surgical Wing

- Hiyalawing Gynae ER

- Hiyalawing Ward A&B, C&D

- Hiyalawing NICU, GOT, LR & LIU

Receiving Station

- Main Laboratory – DH Floor 9 – 4 Receiving stations

Sending Stations

Quantity: 8

Functions:

- Automated dispatch.

- One touch dispatch.

- Sample verification.

- User interface.

- Status indication.

- Tracking capability.

## Receiving Stations

Quantity: 4

Functions:

- Controlled specimen arrival.

- Safe specimen receiving.

- Arrival notification.

- Laboratory workflow integration.

# Section 3– Technical Specifications

## General System Requirements

## 1. Throughput Capacity

### Minimum Mandatory Requirement (Pass/Fail)

The offered system shall demonstrate the capability to transport not less than 1,000 blood collection tubes/specimens per hour under normal operating conditions.

### Evidence Required

- Manufacturer's technical literature.

- Independent validation reports or installed site references.

- Performance calculations.

### Evaluation Method

### Must meet Requirement

## 2. Transport Distance / Network Coverage

### Minimum Mandatory Requirement

The proposed system shall support a transport network covering all designated installation locations within IGMH and Dharumavantha Hospital.

The system shall provide:

- Minimum point-to-point operational distance of 1,800 meters cumulative network capability, or

- Equivalent documented network architecture demonstrating uninterrupted connectivity between all required sending and receiving stations.

### Evidence Required

- System design drawings.

- Network layout.

- Manufacturer specifications.

### Evaluation Method

### Must meet Requirement

## 3. One-Touch Delivery

### Definition

The system shall allow direct insertion of blood collection tubes into the sending station without the use of transport carriers, capsules, secondary containers, or manual repackaging.

### Minimum Requirement

The proposed system shall:

- Accept primary collection tubes directly

- Automatically dispatch samples

- Require no intermediate handling steps

### Evaluation

### Must meet Requirement

## 4. Continuous Transport Operation

### Definition

The system shall support uninterrupted specimen transportation without queue-related system stoppages during peak laboratory demand periods.

### Minimum Requirement

The bidder shall demonstrate:

- Continuous operation 24/7.

- Automatic routing or traffic management.

- No need for manual intervention during normal operation.

### Evidence Required

- Operating principles.

- Installed site references.

- Technical documentation.

### Evaluation

### Must meet Requirement

## 5. Automated Sample Verification

### Definition

The system shall automatically verify specimen compatibility prior to dispatch.

### Minimum Requirement

System shall automatically detect:

- Tube size compatibility.

- Acceptable specimen dimensions.

- Improper loading conditions.

### Evaluation

### Must meet Requirement

## 6. Specimen Protection

### Definition

The system shall maintain specimen integrity during transportation.

### Minimum Requirement

The bidder shall demonstrate:

- Controlled acceleration

- Controlled deceleration

- Shock protection

- Prevention of specimen damage, leakage or hemolysis

### Evidence Required

- Validation studies.

- Manufacturer data.

- Published test reports.

### Evaluation

### Must meet Requirement

## 7. Controllers and Communication Equipment

### Minimum Requirement

The system shall include all necessary:

- Controllers

- Communication modules

- Monitoring hardware

- Network interfaces

- Power supplies

required for operation of the complete transport network.

### Evaluation

### Must meet Requirement

## 8. Software Licensing

### Minimum Requirement

The bidder shall provide all software licenses necessary for:

- System operation

- Monitoring

- Administration

- Maintenance

- Reporting

### Minimum Requirements

The license shall be:

- Perpetual (preferred)

- Non-expiring for operational software

- Transferable to the hospital

- Include all required user licenses

- No recurring subscription fees during warranty period

### Evaluation

### Must meet Requirement

## 9. Local Support Capability

### Minimum Mandatory Requirement

The bidder shall maintain local technical support capability in the Maldives.

### Minimum Requirements

The bidder shall provide:

- At least two certified technical engineer permanently available in Maldives.

- Preventive maintenance capability.

- Spare parts stock availability.

- Remote diagnostic capability.

### Evaluation

### Must meet Requirement

## 10. Training Requirement

### Minimum Mandatory Requirement

The bidder shall train the following users of IGMH.

### Minimum Requirements

The bidder shall provide:

- User Training

  - Laboratory staff - 10

  - Nurses - 10

  - Clinical personnel -10

- Technical Training

- Biomedical staff - 4

- Maintenance staff - 2

- IT staff - 2

### Evaluation

### Must meet Requirement

## 11. System Warranty and Maintenance

### Minimum Mandatory Requirement

The bidder shall train the following users of IGMH.

### Minimum Requirements

The bidder shall provide:

- 24 months comprehensive warranty.

- Preventive maintenance.

- Corrective maintenance.

- Software updates.

- Spare parts support.

### Evaluation

### Must meet Requirement

# Section 4 - Technical Evaluation Matrix

|  | Evaluation Area | Mandatory Requirement |
| --- | --- | --- |
| 1 | Throughput Capacity | Must meet requirement |
| 2 | Network Coverage / Transport Distance | Must meet requirement |
| 3 | One-Touch Delivery | Must meet requirement |
| 4 | Continuous Transport Operation | Must meet requirement |
| 5 | Automated Sample Verification | Must meet requirement |
| 6 | Specimen Protection | Must meet requirement |
| 7 | Controllers & Communication Equipment | Must meet requirement |
| 8 | Software Licensing | Must meet requirement |
| 9 | Local Support Capability | Must meet requirement |
| 10 | Training Requirement | Must meet requirement |
| 11 | Warranty & Maintenance | Must meet requirement |

# Section 5 - Delivery and Implementation Schedule

| Phase | Duration |
| --- | --- |
| Site Survey | 2 Weeks |
| Final Design Approval | 1 Week |
| Manufacturing | 8–10 Weeks |
| Shipping & Clearance | 4–8 Weeks |
| Installation | 4 Weeks |
| Software Configuration | 2 Week |
| Testing & Commissioning | 2 Week |
| Training & Handover | 1 Week |

Maximum Project Duration: 30 Weeks

Section 6 - Compliance Matrix

| No. | Requirement | Comply / Not Comply | Evidence / Page Reference | Remarks |
| --- | --- | --- | --- | --- |
| 1 | Throughput Capacity |  |  |  |
| 2 | Network Coverage / Transport Distance |  |  |  |
| 3 | One-Touch Delivery |  |  |  |
| 4 | Continuous Transport Operation |  |  |  |
| 5 | Automated Sample Verification |  |  |  |
| 6 | Specimen Protection |  |  |  |
| 7 | Controllers & Communication Equipment |  |  |  |
| 8 | Software Licensing |  |  |  |
| 9 | Local Support Capability |  |  |  |
| 10 | Training Requirement |  |  |  |
| 11 | Warranty & Maintenance |  |  |  |
| 12 | Delivery & Implementation Plan |  |  |  |

Conclusion

The procurement of a Point-to-Point Blood Transportation System for Indira Gandhi Memorial Hospital and Dharumavantha Hospital is an important initiative to improve the efficiency, safety, traceability and reliability of laboratory specimen transportation across key clinical service areas. The proposed system is expected to reduce manual handling, minimize delays in specimen movement, improve laboratory turnaround time, and support timely clinical decision-making for patient care.

This bidding document has been prepared to provide clear instructions to bidders, define the required scope of supply and services, establish minimum technical and performance requirements, and set out the evaluation, award, warranty, training, implementation and submission requirements. The document is intended to ensure that all bids are assessed in a transparent, consistent and technically sound manner, in line with the requirements of the Procuring Entity and the applicable procurement procedures.

Accordingly, bidders are required to submit complete, compliant and evidence-based proposals demonstrating their capacity to supply, install, test, commission, train, warrant and maintain the proposed system within the specified timeframe. The successful implementation of this project will strengthen hospital logistics, enhance laboratory workflow, and contribute to improved quality and responsiveness of healthcare services at IGMH and Dharumavantha Hospital.