Section 6 - Schedule of Supply

Contents

1.	List of Goods and Related Services	6-2

2.	Delivery and Completion Schedule	6-3

3.	Technical Specifications	6-4

4.	Drawings	6-5

- List of Goods and Related Services

The Goods and Related Services are grouped in lots. Bidders shall have the option of submitting Bids for one, any combination, or all of the lots. Lots shall not be further subdivided for the purpose of bidding.

| Lot No. : 01 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Lot Name : Industrial and mobile waste shredder |  |  |  |  |
| Item No. | Name of Goods or Related Services | Description | Unit of Measurement | Quantity |
| 01 a | Industrial and mobile waste shredder | Provision of a new, mobile diesel, wheeled industrial waste shredder | Unit | 01 |
| 01 b | Spare parts | Set of spare parts for 2 years | Lump sum | 01 |
| 01 c | Training session | Commissioning, start up and small training session | Lump sum | 01 |

| Lot No. : 02 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Lot Name : Industrial and mobile rotating sieving machine |  |  |  |  |
| Item No. | Name of Goods or Related Services | Description | Unit of Measurement | Quantity |
| 02 a | Industrial and mobile rotating sieving machine | Provision of a new, mobile diesel, wheeled rotating sieving machine | Unit | 01 |
| 02 b | Spare parts | Set of spare parts for 2 years | Lump sum | 01 |

| Lot No. : 03 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Lot Name : Bulldozer |  |  |  |  |
| Item No. | Name of Goods or Related Services | Description | Unit of Measurement | Quantity |
| 3. a | Bulldozer | Provision of a new bulldozer | Unit | 01 |
| 3. b | Spare parts | Set of spare parts for 2 years | Lump sum | 01 |

| Lot No. : 04 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Lot Name : Excavators, wheel loader and waste handler |  |  |  |  |
| Item No. | Name of Goods or Related Services | Description | Unit of Measurement | Quantity |
| 4.1 | Excavators | Provision of a new Excavator | Unit | 02 |
| 4.2 | Wheel loader | Provision of a new, wheel loader | Unit | 01 |
| 4.3 | Waste handler | Provision of a new, wheel waste handler | Unit | 01 |
| 4.4 | Waste handler | Provision of a new, telescopic arm wheel handler | Unit | 01 |
| 4 a | Spare parts | Set of spare parts for 2 years (for excavators and wheel loader | Lump sum | 01 |

| Lot No. : 05 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Lot Name : Arm roll trucks, containers |  |  |  |  |
| Item No. | Name of Goods or Related Services | Description | Unit of Measurement | Quantity |
| 5.1 | Arm roll trucks | Provision of new, arm roll trucks | Unit | 02 |
| 5.2 | Container | Provision of a set of normalized steel container 25 m³ | Unit | 20 |
| 5.3 | Tank trucks | Provision of new, tank trucks | Unit | 01 |
| 5 a | Spare parts | Set of spare parts for 2 years (for trucks) | Lump sum | 01 |

- Delivery and Completion Schedule

The delivery period shall start as of ______________________________________.

| Item No. | Description of Goods or Related Services | Delivery Schedule (Duration) | Location | Required Arrival Date of Goods or Completion Date for Related Services |
| --- | --- | --- | --- | --- |
| 01 | Provision of an industrial and mobile waste shredder including spare parts, commissioning start-up and training | Latest delivery: 6 months after contract award | Male’(Maldives) | At least 2 weeks after delivery |
| 02 | Provision of an industrial and mobile rotating sieving machine including spare parts, commissioning and start-up | Latest delivery: 6 months after contract award | Male’(Maldives) | At least 2 weeks after delivery |
| 03 | Provision of an Bulldozer including spare parts, commissioning and start-up | Latest delivery: 6 months after contract award | Male’(Maldives) | At least 2 weeks after delivery |
| 04 | Provision of two Excavator, one wheel loader, one waste handler and one telescoping arm handler including spare parts, commissioning and start-up | Latest delivery: 6 months after contract award | Male’(Maldives) | At least 2 weeks after delivery |
| 05 | Provision of two arm-roll trucks, a set of containers and one tank truck including spare parts, commissioning and start-up | Latest delivery: 6 months after contract award | Male’(Maldives) | At least 2 weeks after delivery |

- Technical Specifications

| Item No. | Name of Goods or Related Services | Technical Description, Specifications, and Standards |
| --- | --- | --- |
| 01 | Provision of an industrial and mobile waste shredder including spare parts, commissioning start-up and training | Technical description see below |
| 02 | Provision of an industrial and mobile rotating sieving machine including spare parts, commissioning and start-up |  |
| 03 | Provision of an Bulldozer including spare parts, commissioning and start-up |  |
| 04 | Provision of two Excavator, one wheel loader, one waste handler and one telescoping arm handler including spare parts, commissioning and start-up |  |
| 05 | Provision of two arm-roll trucks, a set of containers and one tank truck including spare parts, commissioning and start-up |  |

Detailed Technical Specifications and Standards

These specifications describe the basic requirements for goods. Tenderers are requested to submit with their offers the requisite brochure for the product(s) they intend to supply which contains all the aspects of specification required by the tender.

All the dimensions, weights, size capacities and the like of the equipment to be supplied shall not be outside the tolerances given under below. Deviations from the basic requirements and outside the tolerance, if any and which is/are felt necessary, shall be explained in detail and in writing with the offer, with supporting data such as calculation sheets, the purchase reserves the right to reject the products, if such deviations shall be found critical to the use and operation of the products.

The tenderers are required to present information along with their offers as follows:

- Shortest possible delivery period.

- Information on proper representative and/or workshop for backup service/repair and maintenance including their names and addresses.

Tolerances for the various specification requirements are:

- ± 5% for performance data

- ± 5% for dimensional data

The Tenderers are required to produce manufacturer’s letter, which authorizes them to represent the manufacturer in the Maldives

Detailed Technical Specifications are as follows

Lot 1: Industrial waste shredders

| Specification | Required | Tenderers Response (Y/N) |
| --- | --- | --- |
| A standard production, mobile, diesel-driven wheeled (semi trailer), shredder system latest design, robust construction | yes |  |
| Supplied new | yes |  |
| Most suitable for a maximum variety of waste types including bulky waste, tyres, wood, municipal and commercial waste | yes |  |
| Any information of relevance not included in this specification sheet deemed to be necessary for the enhancement of performance of the equipment shall be included along with the tender response | yes |  |

| Description | Specification | Tenderers response |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Motor rating/power | 280 KW | Yes |  | No |  |
| Fuel Tank | To allow for 8 hrs uninterrupted operations | Yes |  | No |  |
| Continuous flow of material and particle size below 300 mm |  | Yes |  | No |  |
| Easy exchangeable wear parts |  | Yes |  | No |  |
| Continuous flow of material and uniformed particle size |  | Yes |  | No |  |
| Shaft length | Min. 1750 mm | Yes |  | No |  |
| Inlet/aperture dimensions (W x L) | Minimum 1500 x 1750 mm | Yes |  | No |  |
| Discharge belt | Adjustable angle (up to 35°), min horizontal extension 7 m | Yes |  | No |  |
| Over belt magnet (mandatory) |  | Yes |  | No |  |
| Feeding hopper |  | Yes |  | No |  |
| Capacity | Min 50 Tons/hour (MSW) | Yes |  | No |  |
| SERVICING |  |  |  |  |  |
| Wear and spare parts | min 2 year | Yes |  | No |  |

Warranty and manuals

| Specifications | Minimum requirements | Tenderers response |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Each equipment supplied to carry a statement of warranty |  | Yes |  | No |  |
| Warranty duration | min. 12 months or 2000 hrs | Yes |  | No |  |
| All literature minimum in English language |  | Yes |  | No |  |
| Work shop repair manuals supplied |  | Yes |  | No |  |
| Parts catalogue/CD and software supplied |  | Yes |  | No |  |
| handbook and service schedule supplied |  | Yes |  | No |  |

Other requirements

| Specifications | Minimum requirements | Tenderers response |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Local support |  | Yes |  | No |  |
| Training |  | Yes |  | No |  |

Training:

- Technician training must be performed by factory trained personnel with at least one (01) year repair/service experience with proposed machine

- Technician training includes two (02) x four (04) hours on machine session covering two thousand (2000) hour preventative maintenance, diagnostics and fault detection

- All training sessions to be scheduled upon delivery date

- Operator training should be performed by factory trained personnel with at least one (01) year of operational experience with the machine

- Operation training includes two (02) x four (04) hour session on separate days

- Technician training must focus on control systems and electronic diagnosis

- Technician and operator training to be carried out separately.

- Training sessions total : four (04) days x four (04) hour sessions

Lot 2: Industrial rotating sieving machine (rotating drum/Trommel)

| Specification | Required | Tenderers Response (Y/N) |
| --- | --- | --- |
| A standard production, mobile, diesel driven (wheeled, semi-trailer) rotating sieving machine latest design, robust construction | yes |  |
| Supplied new | yes |  |
| Most suitable for the sieving of earth material, with high organic and plastic waste material. | yes |  |
| Any information of relevance not included in this specification sheet deemed to be necessary for the enhancement of performance of the equipment shall be included along with the tender response | yes |  |

| Description | Specification | Tenderers response |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Motor rating/power | 50 KW | Yes |  | No |  |
| Easy exchangeable wear parts |  | Yes |  | No |  |
| Fuel Tank | To allow for 8 hrs uninterrupted operations | Yes |  | No |  |
| Continuous flow of material and uniformed particle size |  | Yes |  | No |  |
| Feeding hopper or belt conveyor |  | Yes |  | No |  |
| Capacity | 100 m³/h | Yes |  | No |  |
| Sieving drum cut off | < 80-100 mm | Yes |  | No |  |
| Discharge belt | Adjustable angle (up to 25%), ground length 5 m | Yes |  | No |  |
| SERVICING |  |  |  |  |  |
| Wear and spare parts | min 2 year | Yes |  | No |  |

Warranty and manuals

| Specifications | Minimum requirements | Tenderers response |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Each equipment supplied to carry a statement of warranty |  | Yes |  | No |  |
| Warranty duration | min. 12 months or 2000 hrs | Yes |  | No |  |
| All literature minimum in English language |  | Yes |  | No |  |
| Work shop repair manuals supplied |  | Yes |  | No |  |
| Parts catalogue/CD and software supplied |  | Yes |  | No |  |
| handbook and service schedule supplied |  | Yes |  | No |  |

Other requirements

| Specifications | Minimum requirements | Tenderers response |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Local support |  | Yes |  | No |  |

Lot 3: Bulldozer

General

Specification

A standard production, diesel powered Motor bulldozer, tracked vehicle, of latest design, robust construction and in current production

Supplied new

Designed for heavy duty specifications capable of operating in harsh conditions and in extremes of mud and dust

Most suitable for filling and compacting of mixed household waste fractions

Any information of relevance not included in this specification sheet deemed to be necessary for the enhancement of performance of the equipment shall be included along with the tender response

Specifications

| Description | Minimum requirements | Tenderers response |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Blade capacity | 3,500 mm | Yes |  | No |  |
| Blade Type | “Universal” (U)-type |  |  |  |  |
| Operating Weight | 20.000 kg |  |  |  |  |
| Net power | 190 kW |  |  |  |  |
| Fuel Tank | 420 l |  |  |  |  |
| Fuel | Diesel |  |  |  |  |
| Minimum Emission standards | Tier III/stage IIIb |  |  |  |  |
| Wear and spare parts | for 2 years |  |  |  |  |

| CABIN |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ROPS certified |  | Yes |  | No |  |
| FOPS certified |  | Yes |  | No |  |
| Tinted safety glass |  | Yes |  | No |  |
| Protection grids |  | Yes |  | No |  |
| Vibration insulated cab suspension |  | Yes |  | No |  |
| Sound suppression (inside Cabin) | max 70 db(A) | Yes |  | No |  |
| Cabin ventilation |  | Yes |  | No |  |
| Automatic heating and air conditioning |  | Yes |  | No |  |
| Air-suspended operator’s seat |  | Yes |  | No |  |
| Pre start cabin heating |  | Yes |  | No |  |
| Head rest |  | Yes |  | No |  |
| Sliding windows right/left |  | Yes |  | No |  |
| Sun shades |  | Yes |  | No |  |
| Inside and outside rear view mirrors right/left |  | Yes |  | No |  |
| Working lights | 4 front and 1 rear | Yes |  | No |  |
| Windshield wiper and washer |  | Yes |  | No |  |
| Sound level at operator´s ear within cab, maximum |  | Yes |  | No |  |

Warranty and manuals

| Specifications | Minimum requirements | Tenderers response |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Each equipment supplied to carry a statement of warranty |  | Yes |  | No |  |
| Warranty duration | min. 12 months or 2000 hrs | Yes |  | No |  |
| All literature minimum in English language |  | Yes |  | No |  |
| Work shop repair manuals supplied |  |  |  |  |  |
| Parts catalogue/CD and software supplied |  | Yes |  | No |  |
| handbook and service schedule supplied |  | Yes |  | No |  |

Lot 4: Excavator, Wheel loader, waste handler and telescoping waste handler

4.1 Excavators

General

Specification

A standard production, diesel powered Motor, tracked excavator, of latest design, robust construction and in current production

Supplied new

Designed for light to heavy duty specifications capable of operating in harsh conditions in extremes of mud and dust

Most suitable for transport and transfer work of household waste fractions and soil/sand. the additional grapple (see section “Additional Device”) is too be supplied only once

| Specifications | Minimum requirements | Tenderers response |  |  |  |
| --- | --- | --- | --- | --- | --- |
| WEIGHT |  |  |  |  |  |
| Operating weight, minimum | 20,000 kg | Yes |  | No |  |
| CONCEPTION |  |  |  |  |  |
| Wear control in hydraulic circuit |  | Yes |  | No |  |
| Master battery disconnect switch |  | Yes |  | No |  |
| Audible back-up alarm |  | Yes |  | No |  |
| Warning horn |  | Yes |  | No |  |
| Towing hitch |  | Yes |  | No |  |
| Anti-theft protection |  | Yes |  | No |  |
| Undercarriage (gauge) | 2,400 mm | Yes |  | No |  |
| ENGINE |  |  |  |  |  |
| Diesel, 4 cylinders, |  | Yes |  | No |  |
| Emissions standard | Tier IIIb |  |  |  |  |
| Performance ISO 3046, minimum | 130 kW (at RPM1,800) | Yes |  | No |  |
| Displacement | 4,0 l | Yes |  | No |  |
| Fuel tank capacity, minimum | 200 l | Yes |  | No |  |
| Fuel bleeding pump |  | Yes |  | No |  |
| Cold starting system |  | Yes |  | No |  |
| Multi disc breaking system |  | Yes |  | No |  |
| Automatic slowdown of the machine when travel direction is changed |  | Yes |  | No |  |
| Bucket shovel |  |  |  |  |  |
| Bucket capacity | 1,5 m³ | Yes |  | No |  |
| Bucket type | Backhoe bucket | Yes |  | No |  |
| linkage | Z-bar linkage |  |  |  |  |
| Lift arm | 2500 mm | Yes |  | No |  |
| Arm type | Standard height arm | Yes |  | No |  |
| Additional device |  |  |  |  |  |
| Grapple | 1,5 m³ | Yes |  | No |  |
| CABIN |  |  |  |  |  |
| ROPS certified |  | Yes |  | No |  |
| FOPS certified |  | Yes |  | No |  |
| Tinted safety glass |  | Yes |  | No |  |
| Protection grids |  | Yes |  | No |  |
| Vibration insulated cab suspension |  | Yes |  | No |  |
| Sound suppression (inside Cabin) | max 70 db(A) | Yes |  | No |  |
| Cabin ventilation |  | Yes |  | No |  |
| Automatic heating and air conditioning |  | Yes |  | No |  |
| Air-suspended operator’s seat |  | Yes |  | No |  |
| Pre start cabin heating |  | Yes |  | No |  |
| Head rest |  | Yes |  | No |  |
| Sliding windows right/left |  | Yes |  | No |  |
| Sun shades |  | Yes |  | No |  |
| Inside and outside rear view mirrors right/left |  | Yes |  | No |  |
| Working lights | 4 front and 1 rear | Yes |  | No |  |
| Windshield wiper and washer |  | Yes |  | No |  |
| Sound level at operator´s ear within cab, maximum |  | Yes |  | No |  |
| SERVICING |  |  |  |  |  |
| Automatic central lubrication system |  | Yes |  | No |  |
| Access walkway steps |  | Yes |  | No |  |
| Environmentally compliant hydraulic oil |  | Yes |  | No |  |
| Wear and spare parts | min 2 year | Yes |  | No |  |
| MISCELLANEOUS |  |  |  |  |  |
| Automatic fire protection system |  | Yes |  | No |  |

Warranty and manuals

| Specifications | Minimum requirements | Tenderers response |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Each vehicle supplied to carry a statement of warranty |  | Yes |  | No |  |
| Warranty duration | min. 12 months or 2000 hrs | Yes |  | No |  |
| All literature minimum in English language |  | Yes |  | No |  |
| Work shop repair manuals supplied |  |  |  |  |  |
| Parts catalogue/CD and software supplied |  |  |  |  |  |
| Drivers handbook and service schedule supplied |  |  |  |  |  |

Other requirements

| Specifications | Minimum requirements | Tenderers response |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Local support |  | Yes |  | No |  |
| Equipment to be painted in | White | Yes |  | No |  |
| Training |  | Yes |  | No |  |

4.2 Wheel loaders

General

Specification

A standard production, diesel powered Motor wheel loader, four wheeled, tandem drive, of latest design, robust construction and in current production

Supplied new

Designed for light to heavy duty specifications capable of operating in harsh conditions in extremes of mud and dust

Most suitable for transport and transfer work of household waste fractions and soil/sand

| Specifications | Minimum requirements | Tenderers response |  |  |  |
| --- | --- | --- | --- | --- | --- |
| WEIGHT |  |  |  |  |  |
| Operating weight, minimum | 12,000 kg | Yes |  | No |  |
| WHEELS |  |  |  |  |  |
| Tires (size) | 20.5 R25, L3 | Yes |  | No |  |
| CONCEPTION |  |  |  |  |  |
| Sound capacity level (external/surround noise), maximum | 110 dB(A) | Yes |  | No |  |
| Steel mudguard |  | Yes |  | No |  |
| Wear control in hydraulic circuit |  | Yes |  | No |  |
| Master battery disconnect switch |  | Yes |  | No |  |
| Audible back-up alarm |  | Yes |  | No |  |
| Warning horn |  | Yes |  | No |  |
| Towing hitch |  | Yes |  | No |  |
| Anti-theft protection |  | Yes |  | No |  |
| ENGINE |  |  |  |  |  |
| Diesel, 4 cylinders, water cooled, turbo charging |  | Yes |  | No |  |
| Emissions standard | Tier III |  |  |  |  |
| Performance ISO 3046, minimum | 100 kW | Yes |  | No |  |
| at RPM | 2,000 | Yes |  | No |  |
| torque | 600 Nm | Yes |  | No |  |
| Displacement | 4,5 l | Yes |  | No |  |
| Fuel tank capacity, minimum | 200 l | Yes |  | No |  |
| Fuel bleeding pump |  | Yes |  | No |  |
| Cold starting system |  | Yes |  | No |  |
| Bucket shovel |  |  |  |  |  |
| Bucket capacity | 2,5 m³ | Yes |  | No |  |
| Bucket type | Reversible Toe plate, General purpose, with teeth | Yes |  | No |  |
| linkage | Z-bar linkage |  |  |  |  |
| Lift arm | 2500 mm | Yes |  | No |  |
| Arm type | Standard height arm | Yes |  | No |  |
| DRIVE |  |  |  |  |  |
| Number of speeds |  |  |  |  |  |
| Max. speed forw. / rev., not less than | 35/6 km/h | Yes |  | No |  |
| hydrostatic breaking system |  | Yes |  | No |  |
| combined inching breaking system |  | Yes |  | No |  |
| Automatic slowdown of the machine when travel direction is changed |  | Yes |  | No |  |
| Change of travel direction without any shock loads to the operator |  | Yes |  | No |  |
| CABIN |  |  |  |  |  |
| ROPS certified |  | Yes |  | No |  |
| FOPS certified |  | Yes |  | No |  |
| Tinted safety glass |  | Yes |  | No |  |
| Protection grids |  | Yes |  | No |  |
| Vibration insulated cab suspension |  | Yes |  | No |  |
| Sound suppression (inside Cabin) | max 70 db(A) | Yes |  | No |  |
| Cabin ventilation |  | Yes |  | No |  |
| Automatic heating and air conditioning |  | Yes |  | No |  |
| Air-suspended operator’s seat |  | Yes |  | No |  |
| Pre start cabin heating |  | Yes |  | No |  |
| Head rest |  | Yes |  | No |  |
| Sliding windows right/left |  | Yes |  | No |  |
| Sun shades |  | Yes |  | No |  |
| Inside and outside rear view mirrors right/left |  | Yes |  | No |  |
| Working lights | 4 front and 1 rear | Yes |  | No |  |
| Windshield wiper and washer |  | Yes |  | No |  |
| Sound level at operator´s ear within cab, maximum |  | Yes |  | No |  |
| SERVICING |  |  |  |  |  |
| Automatic central lubrication system |  | Yes |  | No |  |
| Access walkway steps |  | Yes |  | No |  |
| Environmentally compliant hydraulic oil |  | Yes |  | No |  |
| Wear parts | min 2 year | Yes |  | No |  |
| MISCELLANEOUS |  |  |  |  |  |
| Automatic fire protection system |  | Yes |  | No |  |

Warranty and manuals

| Specifications | Minimum requirements | Tenderers response |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Each vehicle supplied to carry a statement of warranty |  | Yes |  | No |  |
| Warranty duration | min. 12 months or 2000 hrs | Yes |  | No |  |
| All literature minimum in English language |  | Yes |  | No |  |
| Work shop repair manuals supplied |  |  |  |  |  |
| Parts catalogue/CD and software supplied |  |  |  |  |  |
| Drivers handbook and service schedule supplied |  |  |  |  |  |

Other requirements

| Specifications | Minimum requirements | Tenderers response |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Local support |  | Yes |  | No |  |
| Equipment to be painted in | In white | Yes |  | No |  |

4.3 Waste handler

General

Specification

A standard production, diesel powered Motor wheeled waste handler, four wheeled, tandem drive, of latest design, robust construction and in current production

Supplied new

Designed for light to heavy duty specifications capable of operating in harsh summer/hot conditions in extremes of dust and heavy rainfall

Most suitable for handling and transfer work of household waste fractions, metal scrap, bulky waste

| Specifications | Minimum requirements | Tenderers response |  |  |  |
| --- | --- | --- | --- | --- | --- |
| WEIGHT |  |  |  |  |  |
| Operating weight, minimum | 20-23 t | Yes |  | No |  |
| WHEELS |  |  |  |  |  |
| Tires (size) | 10.00-20 or similar | Yes |  | No |  |
| CONCEPTION |  |  |  |  |  |
| Sound capacity level (external/surround noise), maximum | 110 dB(A) | Yes |  | No |  |
| Steel mudguard |  | Yes |  | No |  |
| Wear control in hydraulic circuit |  | Yes |  | No |  |
| Master battery disconnect switch |  | Yes |  | No |  |
| Audible back-up alarm |  | Yes |  | No |  |
| Warning horn |  | Yes |  | No |  |
| Towing hitch |  | Yes |  | No |  |
| Anti-theft protection |  | Yes |  | No |  |
| ENGINE |  |  |  |  |  |
| Diesel, 4 cylinders, water cooled, turbo charging |  | Yes |  | No |  |
| Emissions standard | Tier IIIA |  |  |  |  |
| Performance ISO 3046, minimum | 100 kW/135 HP | Yes |  | No |  |
| Displacement | 4,5 l | Yes |  | No |  |
| Fuel tank capacity, minimum | 200 l | Yes |  | No |  |
| Fuel bleeding pump |  | Yes |  | No |  |
| Cold starting system |  | Yes |  | No |  |
| Equipment |  |  |  |  |  |
| Industrial stick with tipping cylinder and all auxiliaries | 3,50 m | Yes |  | No |  |
| Lifting magnet |  | Yes |  | No |  |
| Sorting grab | 1000 mm. 0.5 m³ | Yes |  | No |  |
| 5-tine grab | Semi closes shells, 0.4 m³ | Yes |  | No |  |
| Log grab | Overlapping tongs 1,0 m² | Yes |  | No |  |
| Central lubrication system |  |  |  |  |  |
| DRIVE |  |  |  |  |  |
| Number of speeds |  |  |  |  |  |
| hydrostatic breaking system |  | Yes |  | No |  |
| combined inching breaking system |  | Yes |  | No |  |
| Automatic slowdown of the machine when travel direction is changed |  | Yes |  | No |  |
| Change of travel direction without any shock loads to the operator |  | Yes |  | No |  |
| CABIN |  |  |  |  |  |
| ROPS certified |  | Yes |  | No |  |
| FOPS certified |  | Yes |  | No |  |
| Tinted safety glass |  | Yes |  | No |  |
| Protection grids |  | Yes |  | No |  |
| Vibration insulated cab suspension |  | Yes |  | No |  |
| Cabin ventilation |  | Yes |  | No |  |
| Automatic heating and air conditioning |  | Yes |  | No |  |
| Air-suspended operator’s seat |  | Yes |  | No |  |
| Head rest |  | Yes |  | No |  |
| Sliding windows right/left |  | Yes |  | No |  |
| Sun shades |  | Yes |  | No |  |
| Inside and outside rear view mirrors right/left |  | Yes |  | No |  |
| Working lights | 4 front and 1 rear | Yes |  | No |  |
| Windshield wiper and washer |  | Yes |  | No |  |
| SERVICING |  |  |  |  |  |
| Automatic central lubrication system |  | Yes |  | No |  |
| Access walkway steps |  | Yes |  | No |  |
| Environmentally compliant hydraulic oil |  | Yes |  | No |  |
| Wear parts | min 2 year | Yes |  | No |  |
| MISCELLANEOUS |  |  |  |  |  |
| Automatic fire protection system |  | Yes |  | No |  |

Warranty and manuals

| Specifications | Minimum requirements | Tenderers response |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Each vehicle supplied to carry a statement of warranty |  | Yes |  | No |  |
| Warranty duration | min. 12 months or 2000 hrs | Yes |  | No |  |
| All literature minimum in English language |  | Yes |  | No |  |
| Work shop repair manuals supplied |  |  |  |  |  |
| Parts catalogue/CD and software supplied |  |  |  |  |  |
| Drivers handbook and service schedule supplied |  |  |  |  |  |

Other requirements

| Specifications | Minimum requirements | Tenderers response |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Local support |  | Yes |  | No |  |
| Equipment to be painted in | in white colour | Yes |  | No |  |
| Training |  | Yes |  | No |  |

4.4 Telescoping arm handler

General

Specification

A standard production, diesel powered Motor telescoping arm handler, four wheeled, tandem drive, of latest design, robust construction and in current production

Supplied new

Designed for light to heavy duty specifications capable of operating in harsh summer/hot conditions in extremes of dust and heavy rainfall

Most suitable for handling wrapped baled waste (round bales) and heaping them up to 8-10 layers, and light material handling purposes

| Specifications | Minimum requirements | Tenderers response |  |  |  |
| --- | --- | --- | --- | --- | --- |
| WEIGHT |  |  |  |  |  |
| Lifting capacity | 3,000 kg | Yes |  | No |  |
| Lifting height | 10 m (at lifting angle 50°, without stabilisers) | Yes |  | No |  |
| WHEELS |  |  |  |  |  |
| Tires (size) | 405/70-24 or similar | Yes |  | No |  |
| CONCEPTION |  |  |  |  |  |
| Sound capacity level (external/surround noise), maximum | 100 dB(A) | Yes |  | No |  |
| Steel mudguard |  | Yes |  | No |  |
| Wear control in hydraulic circuit |  | Yes |  | No |  |
| Master battery disconnect switch |  | Yes |  | No |  |
| Audible back-up alarm |  | Yes |  | No |  |
| Warning horn |  | Yes |  | No |  |
| Towing hitch |  | Yes |  | No |  |
| Anti-theft protection |  | Yes |  | No |  |
| ENGINE |  |  |  |  |  |
| Diesel, 4 cylinders, water cooled, turbo charging |  | Yes |  | No |  |
| Emissions standard | Tier IIIA |  |  |  |  |
| Performance ISO 3046, minimum | 75 kW/100 HP | Yes |  | No |  |
| Displacement | 3,5 l | Yes |  | No |  |
| Fuel tank capacity, minimum | 120 l | Yes |  | No |  |
| Fuel bleeding pump |  | Yes |  | No |  |
| Cold starting system |  | Yes |  | No |  |
| Equipment |  |  |  |  |  |
| Floating fork |  | Yes |  | No |  |
| Hydraulic bale clamp with tubular pipes, mounted on roller bearings, to handle round bales (d 1.2m, l 1.2 m) |  | Yes |  | No |  |
| Central lubrication system |  |  |  |  |  |
| DRIVE |  |  |  |  |  |
| Number of speeds |  |  |  |  |  |
| hydrostatic breaking system |  | Yes |  | No |  |
| combined inching breaking system |  | Yes |  | No |  |
| Automatic slowdown of the machine when travel direction is changed |  | Yes |  | No |  |
| Change of travel direction without any shock loads to the operator |  | Yes |  | No |  |
| CABIN |  |  |  |  |  |
| ROPS certified |  | Yes |  | No |  |
| FOPS certified |  | Yes |  | No |  |
| Tinted safety glass |  | Yes |  | No |  |
| Protection grids |  | Yes |  | No |  |
| Vibration insulated cab suspension |  | Yes |  | No |  |
| Cabin ventilation |  | Yes |  | No |  |
| Automatic heating and air conditioning |  | Yes |  | No |  |
| Air-suspended operator’s seat |  | Yes |  | No |  |
| Head rest |  | Yes |  | No |  |
| Sliding windows right/left |  | Yes |  | No |  |
| Sun shades |  | Yes |  | No |  |
| Inside and outside rear view mirrors right/left |  | Yes |  | No |  |
| Working lights | 4 front and 1 rear | Yes |  | No |  |
| Windshield wiper and washer |  | Yes |  | No |  |
| SERVICING |  |  |  |  |  |
| Automatic central lubrication system |  | Yes |  | No |  |
| Access walkway steps |  | Yes |  | No |  |
| Environmentally compliant hydraulic oil |  | Yes |  | No |  |
| Wear parts | min 2 year | Yes |  | No |  |
| MISCELLANEOUS |  |  |  |  |  |
| Automatic fire protection system |  | Yes |  | No |  |

Warranty and manuals

| Specifications | Minimum requirements | Tenderers response |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Each vehicle supplied to carry a statement of warranty |  | Yes |  | No |  |
| Warranty duration | min. 12 months or 2000 hrs | Yes |  | No |  |
| All literature minimum in English language |  | Yes |  | No |  |
| Work shop repair manuals supplied |  |  |  |  |  |
| Parts catalogue/CD and software supplied |  |  |  |  |  |
| Drivers handbook and service schedule supplied |  |  |  |  |  |

Other requirements

| Specifications | Minimum requirements | Tenderers response |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Local support |  | Yes |  | No |  |
| Equipment to be painted in | in white colour | Yes |  | No |  |
| Training |  | Yes |  | No |  |

Lot 5: Arm-Roll/Hook lift truck (Roll-on/Roll off truck), set of container and tank truck

5.1 Arm Roll/Hook lift truck

General

Specification

A standard production, diesel powered Hook lift truck, (6 x 4), of latest design, robust construction and in current production

Supplied new

Designed for heavy duty specifications capable of operating in harsh warm and heavy rain conditions

Suitable for waste transport

Specifications

| Specifications | Minimum requirements | Tenderers response |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Gross vehicle Mass (GVM) | 28,000 kg | Yes |  | No |  |
| ENGINE |  |  |  |  |  |
| Turbo Diesel, water cooled | 6 cylinder | Yes |  | No |  |
| Emission standards | EURO IIIb or equivalent EPA standard |  |  |  |  |
| Max power | 300 kW | Yes |  | No |  |
| Fuel capacity, minimum | 200 l | Yes |  | No |  |
| DRIVE |  |  |  |  |  |
| Steering method |  |  |  |  |  |
| Transmission |  | Yes |  | No |  |
| Gear | manual | Yes |  | No |  |
| Axes | 6 x 4 |  |  |  |  |
| Tyres | 13 R 22,5 or similar | Yes |  | No |  |
| Wheels | Steel wheel frame | Yes |  | No |  |
| CABIN |  |  |  |  |  |
| Body Type | Normal Cabin | Yes |  | No |  |
| Anti-lock Braking system (ABS) |  | Yes |  | No |  |
| Electronic brake force distribution (EBD) |  | Yes |  | No |  |
| Cruise control |  | Yes |  | No |  |
| Air conditioning |  | Yes |  | No |  |
| Head rest |  | Yes |  | No |  |
| Sliding windows right/left |  | Yes |  | No |  |
| Sun shades |  | Yes |  | No |  |
| Inside and outside rear view mirrors right/left |  | Yes |  | No |  |
| Front and rear mud flaps |  | Yes |  | No |  |
| Superstructure |  |  |  |  |  |
| Roll-on/Roll off/Arm Roll |  | Yes |  | No |  |
| Gravity-locked standard hook, telescopic arm | min height 1500 mm | Yes |  | No |  |
| Dumping angle | Min. 50 | Yes |  | No |  |
| Multilift | suitable for standard container 25 m³ containers with hook | Yes |  | No |  |
| Lifting capacity | 20 tons |  |  |  |  |
| SERVICING |  |  |  |  |  |
| Wear parts | min 2 year | Yes |  | No |  |
| Options |  |  |  |  |  |
| trailer coupling |  |  |  |  |  |

Warranty and manuals

| Specifications | Minimum requirements | Tenderers response |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Each vehicle supplied to carry a statement of warranty |  | Yes |  | No |  |
| Warranty duration | min. 12 months or 2000 hrs | Yes |  | No |  |
| All literature minimum in English language |  | Yes |  | No |  |
| Work shop repair manuals supplied |  |  |  |  |  |
| Parts catalogue/CD and software supplied |  |  |  |  |  |
| Drivers handbook and service schedule supplied |  |  |  |  |  |

Other requirements

| Specifications | Minimum requirements | Tenderers response |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Local support |  | Yes |  | No |  |
| Equipment to be painted in | in white colour | Yes |  | No |  |

6.2 Containers

General

Specification

Standard production, normed ISO roll on/roll of container (hook lift) 25 m³, including tarpaulin or cover

Supplied new, anti-corrosive steel

Suitable for transport of household waste

Any information of relevance not included in this specification sheet deemed to be necessary for the enhancement of performance of the equipment shall be included along with the tender response

Specifications

| Description | Unit | Specification |
| --- | --- | --- |
| capacity | m³ | 25 m³, According ISO norm |
| hook |  | According to ISO norms, height 1500 mm |
| length | mm | 5500 (max.) |
| width | mm | 2.300 (max) |
| height | mm | 2.000 |

Other requirements

| Specifications | Minimum requirements | Tenderers response |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Each container is equipped with a tarpaulin |  | Yes |  | No |  |
| Equipment to be painted in | in white colour | Yes |  | No |  |

5.3 Tank trucks

General

Specification

One standard production, diesel powered Tank truck, of latest design, robust construction and in current production

Supplied new

Designed for water transport and firefighting

Specifications

| Specifications | Minimum requirements | Tenderers response |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Operating weight | 18,000 kg | Yes |  | No |  |  |
| ENGINE |  |  |  |  |  |  |
| Turbo Diesel | 6 cylinder | Yes |  | No |  |  |
| Emission standards | EURO IIIb or similar EPA standards |  |  |  |  |  |
| Max power | 250 kW | Yes |  | No |  |  |
| Fuel capacity, minimum | 300 l | Yes |  | No |  |  |
| DRIVE |  |  |  |  |  |  |
| Steering method |  |  |  |  |  |  |
| Transmission |  | Yes |  | No |  |  |
| Gear | manual | Yes |  | No |  |  |
| Axes and Wheels |  |  |  |  |  |  |
| Axes | 4 x 4 |  |  |  |  |  |
| Tyres | Dimension 13 R22.5 Profile B. M840/L355 | Yes |  | No |  |  |
| Wheels | Steel frame wheels | Yes |  | No |  |  |
| CABIN |  |  |  |  |  |  |
| Body Type | Standard cab | Yes |  | No |  |  |
| Anti-lock Braking system (ABS) |  | Yes |  | No |  |  |
| Electronic brake force distribution (EBD) with Brake assist (BA) |  | Yes |  | No |  |  |
| Cruise control |  | Yes |  | No |  |  |
| Air conditioning |  | Yes |  | No |  |  |
| Head rest |  | Yes |  | No |  |  |
| Sliding windows right/left |  | Yes |  | No |  |  |
| Sun shades |  | Yes |  | No |  |  |
| Inside and outside rear view mirrors right/left |  | Yes |  | No |  |  |
| Front and rear mud flaps |  | Yes |  | No |  |  |
| Tank device |  |  |  |  |  |  |
| Tank Volume | 10.000 l | Yes |  | No |  |  |
|  | PP | Yes |  | No |  |  |
|  | Revision opening DN 500 | Yes |  | No |  |  |
| Purposes | multipurpose: Potable water transport Raw water transport Non aggressive water transport | Yes |  | No |  |  |
| Pump | Autonomous reversible pump approx. 600 l/min. rotary pump mounted on superstructure | Yes |  | No |  |  |
| Pipe | overflow pipe Filling pipe DN 2” with quick coupling “C” for water filling Suction pipe DN 125, 25 m | Yes |  | No |  |  |
| ADDITIONAL DEVICE |  |  |  |  |  |  |
| Dust fighting equipment | Dust Suppression Bar, Deflector sprayer kit includes deflector, ball valve and pipework | Yes |  | No |  |  |
| SERVICING |  |  |  |  |  |  |
| Wear parts | min 2 year | Yes |  | No |  |  |

Warranty and manuals

| Specifications | Minimum requirements | Tenderers response |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Vehicle supplied to carry a statement of warranty |  | Yes |  | No |  |
| Warranty duration | min. 12 months or 2000 hrs | Yes |  | No |  |
| All literature minimum in English language |  | Yes |  | No |  |
| Work shop repair manuals supplied |  |  |  |  |  |
| Parts catalogue/CD and software supplied |  |  |  |  |  |
| Drivers handbook and service schedule supplied |  |  |  |  |  |

Other requirements

| Specifications | Minimum requirements | Tenderers response |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Local support |  | Yes |  | No |  |
| Equipment to be painted in | in white colour | Yes |  | No |  |

- Drawings

The drawings are just for information purposes, deviations are allowed according to technical specifications

Industrial waste shredder

-no drawings or sketches

Drum sieving machine

|  | Mobile, on wheel tracked With feeding hoper and Belt conveyer for sieved fraction (< 50 mm) And Belt conveyer for refused fraction |
| --- | --- |

Bulldozer

Excavator

|  | Additional devices |
| --- | --- |

Wheel Loader

Waste handler

Devices

Telescopic waste handler

Arm roll (Roll on/Roll off truck

Container

Approx. 25 m³: The container should be suitable for lifting with the previous truck and vice versa